BMS Parametrization Data Log

Adaptive Dual EKF Battery Management System

Drexel University — IEEE Research Paper Reference

Date: April 12, 2026

# 1. Dataset

Source: Kollmeyer 2018 — Panasonic NCR18650PF Characterization

Cell: Panasonic NCR18650PF, 2.9Ah nominal, 3.6V nominal

Institution: University of Wisconsin-Madison (Dr. Phillip Kollmeyer)

Temperatures in dataset: 25°C, 10°C, 0°C, -10°C, -20°C

Temperatures modeled: 25°C, 10°C, 0°C (three-temperature model)

HPPC pulse rates: 0.5C, 1C, 2C, 4C, 6C — 5 pulses per SoC step

SoC steps: 100%, 95%, 90%, 80%, 70%...10%, 5%, 0%

Sampling rate: 0.1 s (10 Hz) for HPPC and drive cycles

OCV characterization (external check): C/20 discharge test at 25°C (54.4 hrs, 2453 points) — used for verification only

OCV source for plant model: HPPC settled voltages (same cell as HPPC test — see Section 4.1)

Files used (25°C HPPC): 25degC_5Pulse_HPPC_Pan18650PF.mat

Files used (10°C HPPC): 10degC_5Pulse_HPPC_Pan18650PF.mat

Files used (0°C HPPC): 0degC_5pulse_HPPC_Pan18650PF.mat

Files used (OCV): C20 OCV Test_C20_25dC.mat

# 2. Cell Capacity

Q_nom used: 2.9 Ah (nominal datasheet)

Q from C/20 OCV test: 2.995 Ah (measured)

SoC accounting method: Ah column from dataset (pre-integrated, accounts for inter-pulse discharges)

Note: HPPC file excludes inter-pulse discharges — raw current integration gives wrong SoC. Ah_col field used.

# 3. ECM Architecture

2RC Thevenin equivalent circuit model:

V_cell(t) = OCV(SoC) - I·R0 - V_p1(t) - V_p2(t)

dV_p1/dt = -V_p1/(R1·C1) + I/C1

dV_p2/dt = -V_p2/(R2·C2) + I/C2

dSoC/dt = -I / (3600 · Q_nom)

R0: Ohmic resistance (instantaneous IR drop)

R1, C1: Fast RC branch — electrochemical polarization (tau1 ~ 6-10s)

R2, C2: Slow RC branch — diffusion polarization (tau2 ~ 80s, fixed)

OCV(SoC): Lookup table — raw identified points interpolated to 21-point grid (SoC=0 to 1), derived from HPPC settled voltages

# 4. Parametrization Methodology

## 4.1 OCV Table

Extracted from settled terminal voltage 800s after end of first pulse (1C) in each SoC group. This ensures full RC decay (>10x tau2) and same-cell consistency with the HPPC impedance data. Raw identified points (~11-13 per temperature) are interpolated to a 21-point linear grid (SoC=0 to 1) for use in Simulink.

The C/20 OCV test file was used as an external consistency check only. The plant-model OCV LUT is derived from HPPC settled voltages from the same cell to avoid inter-cell capacity mismatch.

## 4.2 R0 Extraction

Instantaneous ohmic resistance extracted from first pulse of each SoC group (1C rate, clean initial conditions):

R0 = |V_pre - V_post| / I_pulse

V_pre = mean of 10 samples before pulse. V_post = first sample into pulse.

## 4.3 R1, C1 Extraction (lsqcurvefit)

Analytical 1RC fit to 60s post-pulse relaxation window using MATLAB lsqcurvefit. Note: this fits only the fast RC branch (R1/C1). R2/C2 is fixed separately (Section 4.4). The final plant model remains a full 2RC ECM:

V(t) = OCV_settled - I_pulse · R1 · (1 - exp(-t/tau1))

OCV_settled = terminal voltage at t_pulse_end + 800s (true settled OCV).

Bounds: R1 ∈ [0.001, 0.1] Ω, tau1 ∈ [1, 50] s

## 4.4 R2, C2 — Fixed Values

Slow RC time constant (tau2 ~80-300s) cannot be reliably identified from HPPC data — inter-pulse rest periods insufficient for full decay identification. Fixed values used per temperature:

| Temperature | R2 (Ω) | C2 (F) | tau2 (s) |
| --- | --- | --- | --- |
| 25°C | 0.010 | 8000 | 80 |
| 10°C | 0.015 | 8000 | 120 |
| 0°C | 0.020 | 8000 | 160 |

Note: Fixed R2/C2 is the primary motivation for the adaptive parameter EKF layer, which identifies R2/C2 online.

## 4.5 Pulse Selection for LUT

2C pulses selected for LUT fitting (index 3:5:end in pulse_starts array) — best SNR vs dynamics tradeoff. First pulse of each group used for validation to ensure clean (fully-rested) initial conditions.

# 5. Extracted Parameters — 25°C

| SoC | R0 (Ω) | R1 (Ω) | C1 (F) | tau1 (s) | R2 (Ω) | C2 (F) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.000 | 0.0250 | 0.0024 | 414 | 5.9 | 0.010 | 8000 |
| 0.948 | 0.0242 | 0.0024 | 414 | 5.9 | 0.010 | 8000 |
| 0.899 | 0.0221 | 0.0029 | 346 | 5.9 | 0.010 | 8000 |
| 0.802 | 0.0220 | 0.0036 | 277 | 6.3 | 0.010 | 8000 |
| 0.705 | 0.0209 | 0.0042 | 239 | 6.7 | 0.010 | 8000 |
| 0.609 | 0.0210 | 0.0036 | 276 | 6.3 | 0.010 | 8000 |
| 0.512 | 0.0208 | 0.0023 | 444 | 5.1 | 0.010 | 8000 |
| 0.415 | 0.0211 | 0.0025 | 399 | 5.7 | 0.010 | 8000 |
| 0.318 | 0.0220 | 0.0028 | 360 | 6.0 | 0.010 | 8000 |
| 0.270 | 0.0229 | 0.0026 | 385 | 5.8 | 0.010 | 8000 |
| 0.221 | 0.0244 | 0.0027 | 368 | 5.9 | 0.010 | 8000 |
| 0.173 | 0.0263 | 0.0026 | 383 | 5.8 | 0.010 | 8000 |

# 6. Extracted Parameters — 10°C

| SoC | R0 (Ω) | R1 (Ω) | C1 (F) | tau1 (s) | R2 (Ω) | C2 (F) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.000 | 0.0365 | 0.0042 | 239 | 5.9 | 0.015 | 8000 |
| 0.948 | 0.0359 | 0.0042 | 239 | 5.9 | 0.015 | 8000 |
| 0.899 | 0.0338 | 0.0048 | 210 | 6.3 | 0.015 | 8000 |
| 0.802 | 0.0300 | 0.0053 | 188 | 6.7 | 0.015 | 8000 |
| 0.705 | 0.0295 | 0.0058 | 171 | 6.8 | 0.015 | 8000 |
| 0.609 | 0.0303 | 0.0042 | 236 | 6.3 | 0.015 | 8000 |
| 0.512 | 0.0300 | 0.0034 | 297 | 5.8 | 0.015 | 8000 |
| 0.415 | 0.0294 | 0.0036 | 281 | 5.9 | 0.015 | 8000 |
| 0.318 | 0.0323 | 0.0037 | 274 | 5.8 | 0.015 | 8000 |
| 0.270 | 0.0322 | 0.0036 | 276 | 5.7 | 0.015 | 8000 |
| 0.221 | 0.0335 | 0.0041 | 245 | 5.9 | 0.015 | 8000 |

# 7. Extracted Parameters — 0°C

| SoC | R0 (Ω) | R1 (Ω) | C1 (F) | tau1 (s) | R2 (Ω) | C2 (F) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.000 | 0.0503 | 0.0059 | 168 | 5.9 | 0.020 | 8000 |
| 0.948 | 0.0524 | 0.0059 | 169 | 5.9 | 0.020 | 8000 |
| 0.899 | 0.0482 | 0.0065 | 153 | 6.3 | 0.020 | 8000 |
| 0.802 | 0.0434 | 0.0070 | 142 | 6.3 | 0.020 | 8000 |
| 0.705 | 0.0384 | 0.0071 | 140 | 6.5 | 0.020 | 8000 |
| 0.609 | 0.0399 | 0.0050 | 198 | 6.2 | 0.020 | 8000 |
| 0.512 | 0.0419 | 0.0044 | 227 | 6.2 | 0.020 | 8000 |
| 0.415 | 0.0407 | 0.0046 | 218 | 6.2 | 0.020 | 8000 |
| 0.318 | 0.0418 | 0.0047 | 213 | 6.3 | 0.020 | 8000 |
| 0.270 | 0.0424 | 0.0050 | 200 | 6.3 | 0.020 | 8000 |

# 8. Validation Results

## 8.1 Methodology

Validation performed on first pulse of each SoC group (1C rate) — clean initial conditions, no residual RC state. Error computed over relaxation window only (t = 11s to 40s post-pulse). Multi-pulse parallel simulation via MATLAB parsim.

## 8.2 25°C Validation

| SoC | RMS Error (mV) | Pass (<15 mV) |
| --- | --- | --- |
| 1.000 | 4.55 | ✓ |
| 0.952 | 4.96 | ✓ |
| 0.903 | 4.01 | ✓ |
| 0.806 | 7.44 | ✓ |
| 0.710 | 6.07 | ✓ |
| 0.613 | 8.74 | ✓ |
| 0.516 | 9.00 | ✓ |
| 0.419 | 5.28 | ✓ |
| 0.322 | 2.34 | ✓ |
| 0.274 | 1.45 | ✓ |
| 0.225 | 1.71 | ✓ |
| 0.177 | 10.72 | ✓ |
| 0.129 | 12.22 | ✓ |

Mean RMS: 6.04 mV    Max RMS: 12.22 mV    All 13 points pass <15 mV threshold

## 8.3 10°C Validation

| SoC | RMS Error (mV) | Pass (<15 mV) |
| --- | --- | --- |
| 1.000 | 8.07 | ✓ |
| 0.952 | 7.75 | ✓ |
| 0.903 | 7.60 | ✓ |
| 0.806 | 10.23 | ✓ |
| 0.710 | 9.79 | ✓ |
| 0.613 | 11.47 | ✓ |
| 0.516 | 9.96 | ✓ |
| 0.419 | 6.67 | ✓ |
| 0.322 | 5.00 | ✓ |
| 0.274 | 2.41 | ✓ |
| 0.225 | 4.89 | ✓ |

Mean RMS: 7.62 mV    Max RMS: 11.47 mV    All 11 points pass <15 mV threshold

## 8.4 0°C Validation

| SoC | RMS Error (mV) | Pass (<15 mV) |
| --- | --- | --- |
| 1.000 | 15.90 | ~ |
| 0.952 | 12.44 | ✓ |
| 0.903 | 11.53 | ✓ |
| 0.806 | 16.24 | ~ |
| 0.710 | 17.24 | ~ |
| 0.613 | 12.66 | ✓ |
| 0.516 | 11.92 | ✓ |
| 0.419 | 8.73 | ✓ |
| 0.322 | 8.40 | ✓ |
| 0.274 | 7.29 | ✓ |

Mean RMS: 12.24 mV    Max RMS: 17.24 mV    7/10 pass <15 mV. Higher error at 0°C attributed to fixed R2/C2 — addressed by adaptive EKF.

## 8.5 Summary

| Temperature | Mean RMS (mV) | Max RMS (mV) | Points Tested | Pass Rate |
| --- | --- | --- | --- | --- |
| 25°C | 6.04 | 12.22 | 13 | 13/13 (100%) |
| 10°C | 7.62 | 11.47 | 11 | 11/11 (100%) |
| 0°C | 12.24 | 17.24 | 10 | 7/10 (70%) |

# 9. Simulink Plant Model

File: single_cell_battery.slx

Solver: Fixed-step, discrete, Ts = 0.1s

Current input: Step blocks: I_pack (pulse on), I_pack_off (pulse off)

SoC integrator IC: SoC_init (workspace variable)

RC integrators IC: 0 (rested initial condition)

OCV LUT: 21-point 1-D lookup table, soc_bp / ocv_tbl workspace variables

Logged signals: V_sim (terminal voltage), t_sim (time)

Validation harness: MATLAB parsim with 4 parallel workers

# 10. Key Findings & Notes for Paper

1. SoC must be computed from dataset Ah column, not raw current integration — HPPC file excludes inter-pulse discharge steps.

2. OCV table must be derived from same-cell HPPC settled voltages, not a separate C/20 test cell, to avoid inter-cell capacity mismatch.

3. Validation on first pulse of each SoC group (clean initial conditions) is the correct methodology — mid-group pulses have residual RC state that inflates apparent error.

4. R2/C2 cannot be reliably identified from standard HPPC data — inter-pulse rest insufficient for tau2 convergence. Fixed values used, motivating the adaptive EKF parameter identification layer.

5. R0 exhibits expected temperature dependence: 0.021-0.026Ω at 25°C, 0.029-0.037Ω at 10°C, 0.038-0.052Ω at 0°C.

6. tau1 is relatively temperature-insensitive (~6s across all temperatures), consistent with surface kinetics dominating.

7. 25°C and 10°C validation errors are strong for a control-oriented ECM and are suitable for the present paper's modeling scope. 0°C results are acceptable given fixed slow RC — residual error addressed by adaptive EKF.

# 11. Saved Files

| File | Contents |
| --- | --- |
| params_25C_final.mat | soc_lut, R0/R1/C1/R2/C2_lut, soc_bp, ocv_tbl, Q_nom, Ts |
| params_10C_final.mat | soc_lut_10, R0/R1/C1_lut_10, soc_bp_10, ocv_tbl_10, Q_nom, Ts |
| params_0C_final.mat | soc_lut_0, R0/R1/C1_lut_0, soc_bp_0, ocv_tbl_0, Q_nom, Ts |
| params_25C_backup_4_12_26.mat | Backup of earlier single-point 25°C params |
| single_cell_battery.slx | Simulink 2RC ECM plant model |

— End of Data Log — April 12, 2026 —